Setup Guide for FT900 Microsoft Azure IoT demo

I. MICROSOFT AZURE IoT Architecture

BACK-END:  FT900 -> IoT Hub -> Stream Analytics -> CosmosDB (aka DocumentDB)

FRONT-END: Browser -> (Dashboard webpage in) Storage -> CosmosDB  [NG: TODO]

TODO: Integrate the provided Node.JS script to the sample AWS IoT Dashboard at

https://hackernoon.com/building-an-iot-dashboard-using-the-onion-omega-and-amazon-aws-a3520f850c9

II. BACKEND CLOUD Setup Guide:

- Create an IoT Hub resource ‘FT900IoTHub’

Create a resource > 	Internet of Things > IoT Hub

If free tier is not available, select the basic tier.

Creating an IoT Hub will take about 3-5 minutes.

- Create a Device (under IoT Hub) called ‘ft900device1’ – will use SAS token authentication

IoT Hub > IoT devices > Add

Create Device1 using the following details:

DeviceID: ft900device1

Authentication type: Symmetric Key

Auto-generate keys: YES

Click Save button

Device 1 will use SAS token authentication. Copy the generated Primary Key and update ft900device1_sas_azure.pem

- (Optional) Create a Device (under IoT Hub) called ‘ft900device2’ – X509 certificate authentication

This is optional.

Create Device2 using the following details:

DeviceID: ft900device2

Authentication type: x.509 Self-Signed

Primary Thumbprint: d4c46fefa28b5ae07670af5e4dc7973a799bfcc6

Secondary Thumbprint: d4c46fefa28b5ae07670af5e4dc7973a799bfcc6

Click Save button

Device 2 will use X509 certificate authentication. The thumbprint above is retrieved using  the certificate hash of ft900device2_cert.pem.

After creating device1 and device2, both devices should appear as below

At this point, you can now check if FT900 device can connect successfully to Azure IoT using either ft900device1 or ft900device2.

Changes for iot_config.h

#define USE_MQTT_BROKER         MQTT_BROKER_MAZ_IOT

#define USE_MQTT_DEVICE          SAMPLE_DEVICE_1 // or SAMPLE_DEVICE_2

SAMPLE_DEVICE_1 refers to ft900device1 // configured to use SAS authentication

SAMPLE_DEVICE_2 refers to ft900device2 // configured to use X509 authentication

Please confirm that both devices are able to connect successfully. If yes, then proceed to the next step.

- Create a CosmosDB resource ‘ft900iotcosmosdb’

Create a resource > Databases > Azure Cosmos DB

Click “Next: Network” button

Click “Next: Tags” button

Click “Next: Summary” button

Click “Create” button

Creating the CosmosDB database will take about 6 minutes.

- Create a Stream Analytics resource ‘FT900IoTStreamAnalytics’

Create a resource >Analytics > Stream Analytics job

Click “Create” button.

Create the Job Input

Go to Job topology > Inputs > Add stream input > IoT Hub

Add Input alias: FT900StreamInputIoTHub

Click “Save” button

Create the Job Output

Go to Job topology > Outputs  > Add > Cosmos DB

Add Output alias: FT900StreamOutputCosmosDB

Database Create new: timeseriestable

Collection name pattern: mycollections

Click “Save” button

Update the Query

Go to Job topology > Query  > Add > Cosmos DB

Update the query

SELECT

*

INTO

FT900StreamOutputCosmosDB

FROM

FT900StreamInputIoTHub

Click “Save” button

Check that it should look like this below

Click “Start” button > Start

Wait for the Streaming job to start successfully! Status will change from Starting to Running

This will take about 2-3 minutes.

- Verify FT900 MQTT messages are saved in Cosmos DB.

1. Run FT900 IoT Demo for 1 minute.

(make sure MQTT_BROKER_MAZ_IOT and SAMPLE_DEVICE_1 is selected)

2. Go to CosmosDB > Data Explorer

Under timeseriestable > mycollections > Documents

run the following query “SELECT * FROM c”

Verify that the data appears as screenshot below.

These are the packets sent by FT900 device

3. NOTE: To save database memory, change the following values:

Throughput: 400

Time to Live: On (60 seconds) or depending on the dashboard use-case.

Then click Save button

III. FRONTEND CLOUD Setup Guide:

A. Install the prerequisites

1. NodeJS

2. Azure Cosmos API

npm install async –g

npm install @azure/cosmos -g

B. Get the access in key CosmosDB

C. Update and run the Node.JS script  “cosmosdbclient.js”

1. Update the values for

endpoint = "https://ft900iotcosmos.documents.azure.com:443/";

masterKey = " 9eAPFh6cZ4geusIykcyvz1Epy2y95A7e1hqSk13XFdgvEeHoUF92g5YjvJHFhC3EIVBjG0QQnFwfJ5FfsoHcbA==";

databaseId = "timeseriestable";

containerId = "mycollections";

2. Run FT90x for 1-2 minutes

3. Run cosmosdbclient.js using “node cosmosdbclient.js”